# Smart Microgrid P2P Energy Trading System

## Final Year Project - BS Electrical Engineering

## Project Summary

This project implements a Blockchain-based Peer-to-Peer (P2P) Energy Trading System for smart microgrids. The system enables automated energy sharing between a solar-producing household and a neighboring consumer, with all transactions recorded on an immutable blockchain ledger. Additionally, an AI-based load prediction model forecasts future energy generation and consumption patterns.

## Problem Statement

Traditional power grids operate on a centralized model where consumers can only purchase electricity from utility providers. With the rise of distributed energy resources (solar panels, home batteries), prosumers (producer-consumers) often generate excess energy that goes unutilized. Current solutions lack:

- Transparent peer-to-peer trading mechanisms

- Immutable transaction records

- Real-time monitoring and automation

- Predictive analytics for load management

## Objectives

- Design and implement a P2P energy trading system between two households

- Develop automatic excess energy detection and transfer mechanism

- Integrate blockchain technology for transparent transaction recording

- Create a mobile application for real-time monitoring

- Implement AI-based load prediction for optimized energy management

## Methodology

### Hardware Implementation

- Energy Monitoring: PZEM-004T sensors measure voltage, current, power, and energy

- Microcontrollers: ESP32 modules for data acquisition and relay control

- Automatic Transfer: Relay module controlled based on excess energy availability

### Software Implementation

- Smart Contract: Solidity-based contract on Ethereum blockchain (Ganache)

- Backend Server: Node.js with Web3.js for blockchain interaction

- Mobile App: Blynk IoT platform for real-time monitoring

- AI Model: LSTM neural network for 24-hour load prediction

## System Architecture

┌─────────────────────────────────────────────────────────────────────┐
│                        SMART MICROGRID SYSTEM                        │
├─────────────────────────────────────────────────────────────────────┤
│                                                                      │
│   ┌───────────────┐                      ┌───────────────┐          │
│   │   HOUSE A     │   Excess Energy      │   HOUSE B     │          │
│   │   (Producer)  │ ──────────────────▶  │  (Consumer)   │          │
│   │   Solar Panel │      via Relay       │               │          │
│   └───────┬───────┘                      └───────┬───────┘          │
│           │                                      │                   │
│           │ WiFi                                 │ WiFi              │
│           ▼                                      ▼                   │
│   ┌─────────────────────────────────────────────────────┐           │
│   │              BACKEND SERVER (Node.js)                │           │
│   │                    + Web3.js                         │           │
│   └─────────────────────────┬───────────────────────────┘           │
│                             │                                        │
│                             ▼                                        │
│   ┌─────────────────────────────────────────────────────┐           │
│   │         BLOCKCHAIN (Ganache - Ethereum)              │           │
│   │              Smart Contract Layer                    │           │
│   └─────────────────────────────────────────────────────┘           │
│                                                                      │
└─────────────────────────────────────────────────────────────────────┘

## Key Features

| Feature | Description |
| --- | --- |
| Automatic Energy Transfer | Relay activates when solar excess exceeds 100W threshold |
| Blockchain Recording | Every transaction stored with cryptographic hash |
| Real-time Monitoring | Mobile app displays live power readings |
| AI Prediction | LSTM model forecasts next 24 hours of generation/consumption |
| Immutable Ledger | Transaction history cannot be altered or deleted |

## How It Works

Step 1: Energy Generation
House A has a solar panel that generates electricity. The PZEM sensor continuously measures how much power is being produced.

Step 2: Consumption Monitoring
Another sensor monitors how much power House A is consuming for its own appliances (lights, fans, etc.).

Step 3: Excess Calculation
The ESP32 microcontroller calculates: Excess Power = Solar Production - House A Consumption

Step 4: Automatic Transfer Decision
- If excess power ≥ 100W → Relay turns ON → Energy flows to House B - If excess power < 80W → Relay turns OFF → No transfer

Step 5: Blockchain Recording
Every 30 seconds, the transaction data is sent to the backend server, which records it on the blockchain with a unique cryptographic hash. This creates a permanent, tamper-proof record.

Step 6: Real-time Monitoring
Both houses can monitor their energy data in real-time through the Blynk mobile app, including power readings, transfer status, and transaction history.

Step 7: AI Prediction
The collected data is used to train an LSTM neural network that predicts the next 24 hours of solar generation and energy consumption, enabling better energy planning.

### Simple Example

Scenario: House A’s solar panel produces 500W. House A uses 200W for its own needs.

Calculation: Excess = 500W - 200W = 300W

Action: Since 300W > 100W threshold, the relay activates and 300W flows to House B.

Record: Transaction recorded on blockchain: “House A transferred 300W to House B at 2:30 PM”

## Flow Diagrams

### 1. System Architecture

### 2. Energy Flow Diagram

### 3. Decision Flowchart

## Hardware Components

| Component | Quantity | Purpose |
| --- | --- | --- |
| ESP32 Microcontroller | 2 | Data acquisition & control |
| PZEM-004T Sensor | 3 | Energy measurement |
| Relay Module | 1 | Energy transfer switching |
| DC-AC Inverter | 1 | Solar DC to AC conversion |
| Solar Panel | 1 | Energy generation |

## Software Technologies

| Layer | Technology |
| --- | --- |
| Embedded | Arduino (C++) |
| Backend | Node.js, Express.js |
| Blockchain | Solidity, Hardhat, Ganache |
| Blockchain API | Web3.js |
| Mobile App | Blynk IoT |
| Machine Learning | TensorFlow, Keras (LSTM) |

## Expected Outcomes

- Functional prototype demonstrating P2P energy trading between two houses

- Blockchain ledger with immutable transaction records

- Mobile dashboard for real-time monitoring and control

- AI model capable of predicting energy patterns with >85% accuracy

- Complete documentation including wiring diagrams and user guides

## Project Scope

- In Scope: Two-house prototype, local blockchain, mobile monitoring, load prediction

- Out of Scope: Grid integration, billing/payment systems, multi-house scaling

## Conclusion

This project demonstrates the feasibility of blockchain-based P2P energy trading at a household level. By combining IoT sensors, smart contracts, and machine learning, the system provides a transparent, automated, and intelligent solution for distributed energy management in smart microgrids.